Федеральное государственное автономное образовательное

учреждение высшего образования

Национальный исследовательский университет ИТМО

Факультет Программной Инженерии и Компьютерной Техники

Лабораторная работа №2
«Создание взаимосвязанной IP-сети»
по дисциплине
«Администрирование систем и сетей»
Желаем получить по «E» баллу

Выполнили: Ястребов-Амирханов Алекси, Исупов Никита
 Группа: P3432/P3431
Преподаватель: Афанасьев Дмитрий Борисович

Санкт-Петербург,

2026

Оглавление

Цели.	2

Топология сети	2

Процедура конфигурирования	3

Шаг 1 Настройте основные параметры устройств.	3

Шаг 2 Выведите на экран IP-адрес текущего интерфейса и таблицу маршрутизации маршрутизатора.	3

Шаг 3 Настройте IP-адреса для физических интерфейсов	4

Шаг 4 Создайте loopback-интерфейс.	8

Шаг 5 Настройте статические маршруты.	11

Шаг 6 Настройте маршрут от R1 к R2 через R3 в качестве резервного маршрута от LoopBack0 R1 к LoopBack0 R2.	14

Шаг 7 Настройте маршруты по умолчанию для установления связи между интерфейсом LoopBack0 маршрутизатора R1 и интерфейсом LoopBack0 маршрутизатора R2.	18

Вывод	24

Цели.

Лабораторная работа помогает получить практические навыки по изучению следующих тем:

- Процедура настройки IPv4-адреса на интерфейсе

- Функции и значение loopback-интерфейсов

- Принципы генерирования прямых маршрутов

- Процедура настройки статических маршрутов и условия, при которых используются статические маршруты

- Процедура проверки возможности установления соединения сетевого уровня с помощью инструмента ping

- Процедура настройки статических маршрутов и сценарии их применения

Топология сети

Процедура конфигурирования

Шаг 1 Настройте основные параметры устройств.

# Задайте имена устройствам.

R1

<Huawei>system-view Enter system view, return user view with Ctrl+Z. [Huawei]sys [Huawei]sysname R1 [R1]

R2

<Huawei>system-view Enter system view, return user view with Ctrl+Z. [Huawei]sys [Huawei]sysname R2 [R2]

R3

<Huawei>system-view Enter system view, return user view with Ctrl+Z. [Huawei]sys [Huawei]sysname R3 [R3]

Шаг 2 Выведите на экран IP-адрес текущего интерфейса и таблицу маршрутизации маршрутизатора.

# Выведите на экран статус интерфейса на маршрутизаторе (в данном случае на примере R1).

R1

[R1]display ip i [R1]display ip interface b [R1]display ip interface brief *down: administratively down ^down: standby (l): loopback (s): spoofing The number of interface that is UP in Physical is 3 The number of interface that is DOWN in Physical is 1 The number of interface that is UP in Protocol is 1 The number of interface that is DOWN in Protocol is 3 Interface IP Address/Mask Physical Protocol GigabitEthernet0/0/0 unassigned up down GigabitEthernet0/0/1 unassigned up down GigabitEthernet0/0/2 unassigned down down NULL0 unassigned up up(s)

# Выведите на экран таблицу маршрутизации на маршрутизаторе (в данном случае на примере R1)

[R1]disp [R1]display ip [R1]display ip ro [R1]display ip routing-table Route Flags: R - relay, D - download to fib ------------------------------------------------------------------------------ Routing Tables: Public Destinations : 4 Routes : 4 Destination/Mask Proto Pre Cost Flags NextHop Interface 127.0.0.0/8 Direct 0 0 D 127.0.0.1 InLoopBack0 127.0.0.1/32 Direct 0 0 D 127.0.0.1 InLoopBack0 127.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0 255.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0 [R1]

Шаг 3 Настройте IP-адреса для физических интерфейсов

# Настройте IP-адреса для физических интерфейсов на основе следующей таблицы

Табл. 2-1 IP-адреса физических интерфейсов

| Маршрутизатор | Интерфейс | IP-адрес/маска |
| --- | --- | --- |
| R1 | GigabitEthernet0/0/1 | 10.0.13.1/24 |
|  | GigabitEthernet0/0/0 | 10.0.12.1/24 |
| R2 | GigabitEthernet0/0/0 | 10.0.12.2/24 |
|  | GigabitEthernet0/0/2 | 10.0.23.2/24 |
| R3 | GigabitEthernet0/0/1 | 10.0.13.3/24 |
|  | GigabitEthernet0/0/2 | 10.0.23.3/24 |

R1

<R1>sys <R1>system-view Enter system view, return user view with Ctrl+Z. [R1]int [R1]interface G [R1]interface GigabitEthernet 0/0/1 [R1-GigabitEthernet0/0/1]ip add [R1-GigabitEthernet0/0/1]ip address 10.0.13.1 24 Sep 17 2026 02:56:15-08:00 R1 %%01IFNET/4/LINK_STATE(l)[0]:The line protocol IP on the interface GigabitEthernet0/0/1 has entered the UP state. [R1-GigabitEthernet0/0/1]quit [R1]int [R1]interface G [R1]interface GigabitEthernet 0/0/0 [R1-GigabitEthernet0/0/0]ip [R1-GigabitEthernet0/0/0]ip add [R1-GigabitEthernet0/0/0]ip address 10.0.12.1 24 Sep 17 2026 02:57:27-08:00 R1 %%01IFNET/4/LINK_STATE(l)[1]:The line protocol IP on the interface GigabitEthernet0/0/0 has entered the UP state. [R1-GigabitEthernet0/0/0]quit [R1]

R2

<R2>sys <R2>system-view Enter system view, return user view with Ctrl+Z. [R2]in [R2]info-center [R2]interface G [R2]interface GigabitEthernet 0/0/0 [R2-GigabitEthernet0/0/0]ip add [R2-GigabitEthernet0/0/0]ip address 10.0.12.2 24 Sep 17 2026 02:59:00-08:00 R2 %%01IFNET/4/LINK_STATE(l)[0]:The line protocol IP on the interface GigabitEthernet0/0/0 has entered the UP state. [R2-GigabitEthernet0/0/0]quit [R2]in [R2]info-center [R2]interface G [R2]interface GigabitEthernet 0/0/2 [R2-GigabitEthernet0/0/2]ip ad [R2-GigabitEthernet0/0/2]ip address 10.0.23.2 24 Sep 17 2026 02:59:46-08:00 R2 %%01IFNET/4/LINK_STATE(l)[1]:The line protocol IP on the interface GigabitEthernet0/0/2 has entered the UP state. [R2-GigabitEthernet0/0/2]quit [R2]

R3

<R3>sys <R3>system-view Enter system view, return user view with Ctrl+Z. [R3]in [R3]info-center [R3]interface G [R3]interface GigabitEthernet 0/0/1 [R3-GigabitEthernet0/0/1]ip add [R3-GigabitEthernet0/0/1]ip address 10.0.13.3 24 [R3-GigabitEthernet0/0/1] Sep 17 2026 03:00:37-08:00 R3 %%01IFNET/4/LINK_STATE(l)[0]:The line protocol IP on the interface GigabitEthernet0/0/1 has entered the UP state. [R3-GigabitEthernet0/0/1]quit [R3]int [R3]interface G [R3]interface GigabitEthernet 0/0/2 [R3-GigabitEthernet0/0/2]ip add [R3-GigabitEthernet0/0/2]ip address 10.0.23.3 24 Sep 17 2026 03:01:06-08:00 R3 %%01IFNET/4/LINK_STATE(l)[1]:The line protocol IP on the interface GigabitEthernet0/0/2 has entered the UP state. [R3-GigabitEthernet0/0/2]quit [R3]

# Проверьте наличие связи с помощью инструмента ping.

R1

<R1>system-view Enter system view, return user view with Ctrl+Z. [R1]ping 10.0.13.3 PING 10.0.13.3: 56 data bytes, press CTRL_C to break Reply from 10.0.13.3: bytes=56 Sequence=1 ttl=255 time=140 ms Reply from 10.0.13.3: bytes=56 Sequence=2 ttl=255 time=30 ms Reply from 10.0.13.3: bytes=56 Sequence=3 ttl=255 time=20 ms Reply from 10.0.13.3: bytes=56 Sequence=4 ttl=255 time=20 ms Reply from 10.0.13.3: bytes=56 Sequence=5 ttl=255 time=20 ms --- 10.0.13.3 ping statistics --- 5 packet(s) transmitted 5 packet(s) received 0.00% packet loss round-trip min/avg/max = 20/46/140 ms [R1]ping 10.0.12.2 PING 10.0.12.2: 56 data bytes, press CTRL_C to break Reply from 10.0.12.2: bytes=56 Sequence=1 ttl=255 time=110 ms Reply from 10.0.12.2: bytes=56 Sequence=2 ttl=255 time=20 ms Reply from 10.0.12.2: bytes=56 Sequence=3 ttl=255 time=10 ms Reply from 10.0.12.2: bytes=56 Sequence=4 ttl=255 time=10 ms Reply from 10.0.12.2: bytes=56 Sequence=5 ttl=255 time=20 ms --- 10.0.12.2 ping statistics --- 5 packet(s) transmitted 5 packet(s) received 0.00% packet loss round-trip min/avg/max = 10/34/110 ms [R1]

R2

<R2>system-view Enter system view, return user view with Ctrl+Z. [R2]ping 10.0.12.1 PING 10.0.12.1: 56 data bytes, press CTRL_C to break Reply from 10.0.12.1: bytes=56 Sequence=1 ttl=255 time=20 ms Reply from 10.0.12.1: bytes=56 Sequence=2 ttl=255 time=20 ms Reply from 10.0.12.1: bytes=56 Sequence=3 ttl=255 time=20 ms Reply from 10.0.12.1: bytes=56 Sequence=4 ttl=255 time=20 ms Reply from 10.0.12.1: bytes=56 Sequence=5 ttl=255 time=10 ms --- 10.0.12.1 ping statistics --- 5 packet(s) transmitted 5 packet(s) received 0.00% packet loss round-trip min/avg/max = 10/18/20 ms [R2]ping 10.0.23.3 PING 10.0.23.3: 56 data bytes, press CTRL_C to break Reply from 10.0.23.3: bytes=56 Sequence=1 ttl=255 time=40 ms Reply from 10.0.23.3: bytes=56 Sequence=2 ttl=255 time=30 ms Reply from 10.0.23.3: bytes=56 Sequence=3 ttl=255 time=30 ms Reply from 10.0.23.3: bytes=56 Sequence=4 ttl=255 time=20 ms Reply from 10.0.23.3: bytes=56 Sequence=5 ttl=255 time=10 ms --- 10.0.23.3 ping statistics --- 5 packet(s) transmitted 5 packet(s) received 0.00% packet loss round-trip min/avg/max = 10/26/40 ms [R2]

R3

<R3>system-view Enter system view, return user view with Ctrl+Z. [R3]ping 10.0.13.1 PING 10.0.13.1: 56 data bytes, press CTRL_C to break Reply from 10.0.13.1: bytes=56 Sequence=1 ttl=255 time=30 ms Reply from 10.0.13.1: bytes=56 Sequence=2 ttl=255 time=30 ms Reply from 10.0.13.1: bytes=56 Sequence=3 ttl=255 time=20 ms Reply from 10.0.13.1: bytes=56 Sequence=4 ttl=255 time=20 ms Reply from 10.0.13.1: bytes=56 Sequence=5 ttl=255 time=20 ms --- 10.0.13.1 ping statistics --- 5 packet(s) transmitted 5 packet(s) received 0.00% packet loss round-trip min/avg/max = 20/24/30 ms [R3]ping 10.0.23.2 PING 10.0.23.2: 56 data bytes, press CTRL_C to break Reply from 10.0.23.2: bytes=56 Sequence=1 ttl=255 time=30 ms Reply from 10.0.23.2: bytes=56 Sequence=2 ttl=255 time=20 ms Reply from 10.0.23.2: bytes=56 Sequence=3 ttl=255 time=20 ms Reply from 10.0.23.2: bytes=56 Sequence=4 ttl=255 time=20 ms Reply from 10.0.23.2: bytes=56 Sequence=5 ttl=255 time=20 ms --- 10.0.23.2 ping statistics --- 5 packet(s) transmitted 5 packet(s) received 0.00% packet loss round-trip min/avg/max = 20/22/30 ms [R3]

# Выведите на экран таблицу маршрутизации R1.

R1

[R1]display ip routing-table Route Flags: R - relay, D - download to fib ------------------------------------------------------------------------------ Routing Tables: Public Destinations : 10 Routes : 10 Destination/Mask Proto Pre Cost Flags NextHop Interface 10.0.12.0/24 Direct 0 0 D 10.0.12.1 GigabitEthernet0/0/0 10.0.12.1/32 Direct 0 0 D 127.0.0.1 GigabitEthernet0/0/0 10.0.12.255/32 Direct 0 0 D 127.0.0.1 GigabitEthernet0/0/0 10.0.13.0/24 Direct 0 0 D 10.0.13.1 GigabitEthernet0/0/1 10.0.13.1/32 Direct 0 0 D 127.0.0.1 GigabitEthernet0/0/1 10.0.13.255/32 Direct 0 0 D 127.0.0.1 GigabitEthernet0/0/1 127.0.0.0/8 Direct 0 0 D 127.0.0.1 InLoopBack0 127.0.0.1/32 Direct 0 0 D 127.0.0.1 InLoopBack0 127.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0 255.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0

Шаг 4 Создайте loopback-интерфейс.

# Настройте loopback-интерфейс в соответствии со следующей таблицей.

Табл. 2-2 IP-адреса loopback-интерфейсов

| Маршрутизатор | Интерфейс | IP-адрес/маска |
| --- | --- | --- |
| R1 | LoopBack0 | 10.0.1.1/32 |
| R2 | LoopBack0 | 10.0.1.2/32 |
| R3 | LoopBack0 | 10.0.1.3/32 |

R1

<R1>system-view Enter system view, return user view with Ctrl+Z. [R1]int [R1]interface L [R1]interface LoopBack0 [R1-LoopBack0]ip add [R1-LoopBack0]ip address 10.0.1.1 32 [R1-LoopBack0]

R2

<R2>system-view Enter system view, return user view with Ctrl+Z. [R2]in [R2]info-center [R2]interface l [R2]interface LoopBack0 [R2-LoopBack0]ip add [R2-LoopBack0]ip address 10.0.1.2 32 [R2-LoopBack0]

R3

<R3>system-view Enter system view, return user view with Ctrl+Z. [R3]in [R3]info-center [R3]interface l [R3]interface LoopBack0 [R3-LoopBack0]ip a [R3-LoopBack0]ip address 10.0.1.3 32 [R3-LoopBack0]

# Выведите на экран таблицу маршрутизации на маршрутизаторе

R1

[R1-LoopBack0]display ip routing-table Route Flags: R - relay, D - download to fib ------------------------------------------------------------------------------ Routing Tables: Public Destinations : 11 Routes : 11 Destination/Mask Proto Pre Cost Flags NextHop Interface 10.0.1.1/32 Direct 0 0 D 127.0.0.1 LoopBack0 10.0.12.0/24 Direct 0 0 D 10.0.12.1 GigabitEthernet 0/0/0 10.0.12.1/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/0 10.0.12.255/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/0 10.0.13.0/24 Direct 0 0 D 10.0.13.1 GigabitEthernet 0/0/1 10.0.13.1/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/1 10.0.13.255/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/1 127.0.0.0/8 Direct 0 0 D 127.0.0.1 InLoopBack0 127.0.0.1/32 Direct 0 0 D 127.0.0.1 InLoopBack0 127.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0 255.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0 [R1-LoopBack0]

R2

[R2-LoopBack0]display ip routing-table Route Flags: R - relay, D - download to fib ------------------------------------------------------------------------------ Routing Tables: Public Destinations : 11 Routes : 11 Destination/Mask Proto Pre Cost Flags NextHop Interface 10.0.1.2/32 Direct 0 0 D 127.0.0.1 LoopBack0 10.0.12.0/24 Direct 0 0 D 10.0.12.2 GigabitEthernet 0/0/0 10.0.12.2/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/0 10.0.12.255/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/0 10.0.23.0/24 Direct 0 0 D 10.0.23.2 GigabitEthernet 0/0/2 10.0.23.2/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/2 10.0.23.255/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/2 127.0.0.0/8 Direct 0 0 D 127.0.0.1 InLoopBack0 127.0.0.1/32 Direct 0 0 D 127.0.0.1 InLoopBack0 127.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0 255.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0 [R2-LoopBack0]

R3

[R3-LoopBack0]display ip routing-table Route Flags: R - relay, D - download to fib ------------------------------------------------------------------------------ Routing Tables: Public Destinations : 11 Routes : 11 Destination/Mask Proto Pre Cost Flags NextHop Interface 10.0.1.3/32 Direct 0 0 D 127.0.0.1 LoopBack0 10.0.13.0/24 Direct 0 0 D 10.0.13.3 GigabitEthernet 0/0/1 10.0.13.3/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/1 10.0.13.255/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/1 10.0.23.0/24 Direct 0 0 D 10.0.23.3 GigabitEthernet 0/0/2 10.0.23.3/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/2 10.0.23.255/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/2 127.0.0.0/8 Direct 0 0 D 127.0.0.1 InLoopBack0 127.0.0.1/32 Direct 0 0 D 127.0.0.1 InLoopBack0 127.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0 255.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0 [R3-LoopBack0]

# Проверьте наличие связи между loopback-интерфейсами

R1

[R1]ping -a 10.0.1.1 10.0.1.2 PING 10.0.1.2: 56 data bytes, press CTRL_C to break Request time out Request time out Request time out Request time out Request time out --- 10.0.1.2 ping statistics --- 5 packet(s) transmitted 0 packet(s) received 100.00% packet loss [R1]

R2

[R2]ping -a 10.0.1.2 10.0.1.1 PING 10.0.1.1: 56 data bytes, press CTRL_C to break Request time out Request time out Request time out Request time out Request time out --- 10.0.1.1 ping statistics --- 5 packet(s) transmitted 0 packet(s) received 100.00% packet loss [R2]

R3

[R3]ping -a 10.0.1.3 10.0.1.1 PING 10.0.1.1: 56 data bytes, press CTRL_C to break Request time out Request time out Request time out Request time out Request time out --- 10.0.1.1 ping statistics --- 5 packet(s) transmitted 0 packet(s) received 100.00% packet loss [R3]

* На данный момент у маршрутизатора нет маршрута к IP-адресу пункта назначения. Таким образом, операция ping не выполняется.

Шаг 5 Настройте статические маршруты.

# На маршрутизаторе R1 настройте маршрут к интерфейсам LoopBack0 маршрутизаторов R2 и R3.

R1

<R1>system-view Enter system view, return user view with Ctrl+Z. [R1]ip ro [R1]ip route [R1]ip route-static 10.0.1.2 32 10.0.12.2 [R1]ip route-static 10.0.1.3 32 10.0.13.3

# Выведите на экран таблицу маршрутизации R1.

R1

[R1]display ip routing-table Route Flags: R - relay, D - download to fib ------------------------------------------------------------------------------ Routing Tables: Public Destinations : 13 Routes : 13 Destination/Mask Proto Pre Cost Flags NextHop Interface 10.0.1.1/32 Direct 0 0 D 127.0.0.1 LoopBack0 10.0.1.2/32 Static 60 0 RD 10.0.12.2 GigabitEthernet 0/0/0 10.0.1.3/32 Static 60 0 RD 10.0.13.3 GigabitEthernet 0/0/1 10.0.12.0/24 Direct 0 0 D 10.0.12.1 GigabitEthernet 0/0/0 10.0.12.1/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/0 10.0.12.255/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/0 10.0.13.0/24 Direct 0 0 D 10.0.13.1 GigabitEthernet 0/0/1 10.0.13.1/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/1 10.0.13.255/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/1 127.0.0.0/8 Direct 0 0 D 127.0.0.1 InLoopBack0 127.0.0.1/32 Direct 0 0 D 127.0.0.1 InLoopBack0 127.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0 255.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0 [R1]

# Проверьте возможность установления связи.

R1

[R1]ping -a 10.0.1.1 10.0.1.2 PING 10.0.1.2: 56 data bytes, press CTRL_C to break Request time out Request time out Request time out Request time out Request time out --- 10.0.1.2 ping statistics --- 5 packet(s) transmitted 0 packet(s) received 100.00% packet loss [R1]

* Связь с интерфейсом LoopBack0 маршрутизатора R2 по-прежнему отсутствует, поскольку у R2 нет маршрута к интерфейсу LoopBack0 маршрутизатора R1.

# На R2 добавьте маршрут к интерфейсу LoopBack0 маршрутизатора R1.

R2

[R2]ip route-static 10.0.1.1 32 10.0.12.1

# Проверьте возможность установления связи.

[R1]ping -a 10.0.1.1 10.0.1.2 PING 10.0.1.2: 56 data bytes, press CTRL_C to break Reply from 10.0.1.2: bytes=56 Sequence=1 ttl=255 time=10 ms Reply from 10.0.1.2: bytes=56 Sequence=2 ttl=255 time=30 ms Reply from 10.0.1.2: bytes=56 Sequence=3 ttl=255 time=30 ms Reply from 10.0.1.2: bytes=56 Sequence=4 ttl=255 time=30 ms Reply from 10.0.1.2: bytes=56 Sequence=5 ttl=255 time=20 ms --- 10.0.1.2 ping statistics --- 5 packet(s) transmitted 5 packet(s) received 0.00% packet loss round-trip min/avg/max = 10/24/30 ms [R1]

LoopBack0 на R1 может взаимодействовать с LoopBack0 на R2.

# Настройте другие необходимые маршруты.

R2

[R2]ip route-static 10.0.1.3 32 10.0.23.3

R3

[R3]ip route-static 10.0.1.1 32 10.0.13.1 [R3]ip route-static 10.0.1.2 32 10.0.23.2

# Проверьте возможность установления связи между интерфейсами LoopBack0 маршрутизаторов

R2

[R2]ping -a 10.0.1.2 10.0.1.3 PING 10.0.1.3: 56 data bytes, press CTRL_C to break Reply from 10.0.1.3: bytes=56 Sequence=1 ttl=255 time=40 ms Reply from 10.0.1.3: bytes=56 Sequence=2 ttl=255 time=30 ms Reply from 10.0.1.3: bytes=56 Sequence=3 ttl=255 time=30 ms Reply from 10.0.1.3: bytes=56 Sequence=4 ttl=255 time=10 ms Reply from 10.0.1.3: bytes=56 Sequence=5 ttl=255 time=20 ms --- 10.0.1.3 ping statistics --- 5 packet(s) transmitted 5 packet(s) received 0.00% packet loss round-trip min/avg/max = 10/26/40 ms [R2]

R3

[R3]ping -a 10.0.1.3 10.0.1.1 PING 10.0.1.1: 56 data bytes, press CTRL_C to break Reply from 10.0.1.1: bytes=56 Sequence=1 ttl=255 time=20 ms Reply from 10.0.1.1: bytes=56 Sequence=2 ttl=255 time=30 ms Reply from 10.0.1.1: bytes=56 Sequence=3 ttl=255 time=20 ms Reply from 10.0.1.1: bytes=56 Sequence=4 ttl=255 time=40 ms Reply from 10.0.1.1: bytes=56 Sequence=5 ttl=255 time=30 ms --- 10.0.1.1 ping statistics --- 5 packet(s) transmitted 5 packet(s) received 0.00% packet loss round-trip min/avg/max = 20/28/40 ms [R3]ping -a 10.0.1.3 10.0.1.2 PING 10.0.1.2: 56 data bytes, press CTRL_C to break Reply from 10.0.1.2: bytes=56 Sequence=1 ttl=255 time=30 ms Reply from 10.0.1.2: bytes=56 Sequence=2 ttl=255 time=10 ms Reply from 10.0.1.2: bytes=56 Sequence=3 ttl=255 time=20 ms Reply from 10.0.1.2: bytes=56 Sequence=4 ttl=255 time=20 ms Reply from 10.0.1.2: bytes=56 Sequence=5 ttl=255 time=20 ms --- 10.0.1.2 ping statistics --- 5 packet(s) transmitted 5 packet(s) received 0.00% packet loss round-trip min/avg/max = 10/20/30 ms [R3]

Шаг 6 Настройте маршрут от R1 к R2 через R3 в качестве резервного маршрута от LoopBack0 R1 к LoopBack0 R2.

# Настройте статические маршруты на R1 и R2.

R1

[R1]ip route-static 10.0.1.2 32 10.0.13.3 preference 100

R2

[R2]ip route-static 10.0.1.1 32 10.0.23.3 preference 100

# Выведите на экран таблицы маршрутизации R1 и R2.

R1

[R1]display ip routing-table Route Flags: R - relay, D - download to fib ------------------------------------------------------------------------------ Routing Tables: Public Destinations : 13 Routes : 13 Destination/Mask Proto Pre Cost Flags NextHop Interface 10.0.1.1/32 Direct 0 0 D 127.0.0.1 LoopBack0 10.0.1.2/32 Static 60 0 RD 10.0.12.2 GigabitEthernet 0/0/0 10.0.1.3/32 Static 60 0 RD 10.0.13.3 GigabitEthernet 0/0/1 10.0.12.0/24 Direct 0 0 D 10.0.12.1 GigabitEthernet 0/0/0 10.0.12.1/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/0 10.0.12.255/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/0 10.0.13.0/24 Direct 0 0 D 10.0.13.1 GigabitEthernet 0/0/1 10.0.13.1/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/1 10.0.13.255/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/1 127.0.0.0/8 Direct 0 0 D 127.0.0.1 InLoopBack0 127.0.0.1/32 Direct 0 0 D 127.0.0.1 InLoopBack0 127.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0 255.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0 [R1]

R2

[R2]display ip routing-table Route Flags: R - relay, D - download to fib ------------------------------------------------------------------------------ Routing Tables: Public Destinations : 13 Routes : 13 Destination/Mask Proto Pre Cost Flags NextHop Interface 10.0.1.1/32 Static 60 0 RD 10.0.12.1 GigabitEthernet 0/0/0 10.0.1.2/32 Direct 0 0 D 127.0.0.1 LoopBack0 10.0.1.3/32 Static 60 0 RD 10.0.23.3 GigabitEthernet 0/0/2 10.0.12.0/24 Direct 0 0 D 10.0.12.2 GigabitEthernet 0/0/0 10.0.12.2/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/0 10.0.12.255/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/0 10.0.23.0/24 Direct 0 0 D 10.0.23.2 GigabitEthernet 0/0/2 10.0.23.2/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/2 10.0.23.255/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/2 127.0.0.0/8 Direct 0 0 D 127.0.0.1 InLoopBack0 127.0.0.1/32 Direct 0 0 D 127.0.0.1 InLoopBack0 127.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0 255.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0 [R2]

* Статический маршрут со значением предпочтения 100 не был добавлен в таблицу маршрутизации.

# Отключите интерфейс GigabitEthernet0/0/0 на маршрутизаторах R1 и R2, чтобы сделать недействительным маршрут с наивысшим приоритетом.

R1

[R1]interface GigabitEthernet 0/0/0 [R1-GigabitEthernet0/0/0]shut [R1-GigabitEthernet0/0/0]shutdown Sep 17 2026 04:41:45-08:00 R1 %%01IFPDT/4/IF_STATE(l)[0]:Interface GigabitEthern et0/0/0 has turned into DOWN state. [R1-GigabitEthernet0/0/0] [R1-GigabitEthernet0/0/0] Sep 17 2026 04:41:46-08:00 R1 %%01IFNET/4/LINK_STATE(l)[1]:The line protocol IP on the interface GigabitEthernet0/0/0 has entered the DOWN state. [R1-GigabitEthernet0/0/0]

R2

[R2] Sep 17 2026 04:41:52-08:00 R2 %%01IFPDT/4/IF_STATE(l)[0]:Interface GigabitEthern et0/0/0 has turned into DOWN state. [R2] Sep 17 2026 04:41:52-08:00 R2 %%01IFNET/4/LINK_STATE(l)[1]:The line protocol IP on the interface GigabitEthernet0/0/0 has entered the DOWN state. [R2]

# Выведите на экран таблицы маршрутизации на R1 и R2. Из командного вывода видно, что маршруты с более низким приоритетом активируются, когда маршруты с более высоким приоритетом становятся недействительными.

R1

[R1]display ip routing-table Route Flags: R - relay, D - download to fib ------------------------------------------------------------------------------ Routing Tables: Public Destinations : 10 Routes : 10 Destination/Mask Proto Pre Cost Flags NextHop Interface 10.0.1.1/32 Direct 0 0 D 127.0.0.1 LoopBack0 10.0.1.2/32 Static 100 0 RD 10.0.13.3 GigabitEthernet 0/0/1 10.0.1.3/32 Static 60 0 RD 10.0.13.3 GigabitEthernet 0/0/1 10.0.13.0/24 Direct 0 0 D 10.0.13.1 GigabitEthernet 0/0/1 10.0.13.1/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/1 10.0.13.255/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/1 127.0.0.0/8 Direct 0 0 D 127.0.0.1 InLoopBack0 127.0.0.1/32 Direct 0 0 D 127.0.0.1 InLoopBack0 127.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0 255.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0 [R1]

R2

[R2]display ip routing-table Route Flags: R - relay, D - download to fib ------------------------------------------------------------------------------ Routing Tables: Public Destinations : 10 Routes : 10 Destination/Mask Proto Pre Cost Flags NextHop Interface 10.0.1.1/32 Static 100 0 RD 10.0.23.3 GigabitEthernet 0/0/2 10.0.1.2/32 Direct 0 0 D 127.0.0.1 LoopBack0 10.0.1.3/32 Static 60 0 RD 10.0.23.3 GigabitEthernet 0/0/2 10.0.23.0/24 Direct 0 0 D 10.0.23.2 GigabitEthernet 0/0/2 10.0.23.2/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/2 10.0.23.255/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/2 127.0.0.0/8 Direct 0 0 D 127.0.0.1 InLoopBack0 127.0.0.1/32 Direct 0 0 D 127.0.0.1 InLoopBack0 127.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0 255.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0 [R2]

В этом случае исходный статический маршрут становится недействительным и активируется статический маршрут с более низким приоритетом.

# Проверьте возможность установления связи.

R1

[R1]ping -a 10.0.1.1 10.0.1.2 PING 10.0.1.2: 56 data bytes, press CTRL_C to break Reply from 10.0.1.2: bytes=56 Sequence=1 ttl=254 time=50 ms Reply from 10.0.1.2: bytes=56 Sequence=2 ttl=254 time=30 ms Reply from 10.0.1.2: bytes=56 Sequence=3 ttl=254 time=30 ms Reply from 10.0.1.2: bytes=56 Sequence=4 ttl=254 time=20 ms Reply from 10.0.1.2: bytes=56 Sequence=5 ttl=254 time=40 ms --- 10.0.1.2 ping statistics --- 5 packet(s) transmitted 5 packet(s) received 0.00% packet loss round-trip min/avg/max = 20/34/50 ms [R1]

R2

[R2]ping -a 10.0.1.2 10.0.1.1 PING 10.0.1.1: 56 data bytes, press CTRL_C to break Reply from 10.0.1.1: bytes=56 Sequence=1 ttl=254 time=20 ms Reply from 10.0.1.1: bytes=56 Sequence=2 ttl=254 time=40 ms Reply from 10.0.1.1: bytes=56 Sequence=3 ttl=254 time=20 ms Reply from 10.0.1.1: bytes=56 Sequence=4 ttl=254 time=30 ms Reply from 10.0.1.1: bytes=56 Sequence=5 ttl=254 time=30 ms --- 10.0.1.1 ping statistics --- 5 packet(s) transmitted 5 packet(s) received 0.00% packet loss round-trip min/avg/max = 20/28/40 ms [R2]

# Выполните трассировку маршрута, по которому передаются пакеты данных.

R1

[R1]tracert -a 10.0.1.1 10.0.1.2 traceroute to 10.0.1.2(10.0.1.2), max hops: 30 ,packet length: 40,press CTRL_C to break 1 10.0.13.3 10 ms 10 ms 10 ms 2 10.0.23.2 30 ms 30 ms 20 ms [R1]

R2

[R2]tracert -a 10.0.1.2 10.0.1.1 traceroute to 10.0.1.1(10.0.1.1), max hops: 30 ,packet length: 40,press CTRL_C to break 1 10.0.23.3 20 ms 20 ms 20 ms 2 10.0.13.1 50 ms 30 ms 30 ms [R2]

Шаг 7 Настройте маршруты по умолчанию для установления связи между интерфейсом LoopBack0 маршрутизатора R1 и интерфейсом LoopBack0 маршрутизатора R2.

# Включите интерфейсы и удалите настроенные маршруты.

R1

[R1]interface GigabitEthernet 0/0/0 [R1-GigabitEthernet0/0/0]un [R1-GigabitEthernet0/0/0]undo shu [R1-GigabitEthernet0/0/0]undo shutdown [R1-GigabitEthernet0/0/0] Sep 17 2026 04:53:08-08:00 R1 %%01IFPDT/4/IF_STATE(l)[2]:Interface GigabitEthern et0/0/0 has turned into UP state. [R1-GigabitEthernet0/0/0] Sep 17 2026 04:53:08-08:00 R1 %%01IFNET/4/LINK_STATE(l)[3]:The line protocol IP on the interface GigabitEthernet0/0/0 has entered the UP state. [R1-GigabitEthernet0/0/0] [R1-GigabitEthernet0/0/0]quit [R1]undo ip route-static 10.0.1.2 32 10.0.12.2 [R1]undo ip route-static 10.0.1.2 32 10.0.13.3 preference 100

# Выведите на экран таблицу маршрутизации R1.

R1

[R1]display ip routing-table Route Flags: R - relay, D - download to fib ------------------------------------------------------------------------------ Routing Tables: Public Destinations : 12 Routes : 12 Destination/Mask Proto Pre Cost Flags NextHop Interface 10.0.1.1/32 Direct 0 0 D 127.0.0.1 LoopBack0 10.0.1.3/32 Static 60 0 RD 10.0.13.3 GigabitEthernet 0/0/1 10.0.12.0/24 Direct 0 0 D 10.0.12.1 GigabitEthernet 0/0/0 10.0.12.1/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/0 10.0.12.255/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/0 10.0.13.0/24 Direct 0 0 D 10.0.13.1 GigabitEthernet 0/0/1 10.0.13.1/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/1 10.0.13.255/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/1 127.0.0.0/8 Direct 0 0 D 127.0.0.1 InLoopBack0 127.0.0.1/32 Direct 0 0 D 127.0.0.1 InLoopBack0 127.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0 255.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0 [R1]

R1 не имеет маршрута к LoopBack0 (10.1.1.2/32) маршрутизатора R2.

# Настройте маршрут по умолчанию на R1

R1

[R1]ip route-static 0.0.0.0 0 10.0.12.2

# Выведите на экран таблицу маршрутизации R1.

R1

[R1]display ip routing-table Route Flags: R - relay, D - download to fib ------------------------------------------------------------------------------ Routing Tables: Public Destinations : 13 Routes : 13 Destination/Mask Proto Pre Cost Flags NextHop Interface 0.0.0.0/0 Static 60 0 RD 10.0.12.2 GigabitEthernet 0/0/0 10.0.1.1/32 Direct 0 0 D 127.0.0.1 LoopBack0 10.0.1.3/32 Static 60 0 RD 10.0.13.3 GigabitEthernet 0/0/1 10.0.12.0/24 Direct 0 0 D 10.0.12.1 GigabitEthernet 0/0/0 10.0.12.1/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/0 10.0.12.255/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/0 10.0.13.0/24 Direct 0 0 D 10.0.13.1 GigabitEthernet 0/0/1 10.0.13.1/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/1 10.0.13.255/32 Direct 0 0 D 127.0.0.1 GigabitEthernet 0/0/1 127.0.0.0/8 Direct 0 0 D 127.0.0.1 InLoopBack0 127.0.0.1/32 Direct 0 0 D 127.0.0.1 InLoopBack0 127.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0 255.255.255.255/32 Direct 0 0 D 127.0.0.1 InLoopBack0 [R1]

Маршрут по умолчанию был активирован

# Проверьте наличие связи между LoopBack0 маршрутизатора R1 и LoopBack0 маршрутизатора R2.

R1

[R1]ping -a 10.0.1.1 10.0.1.2 PING 10.0.1.2: 56 data bytes, press CTRL_C to break Reply from 10.0.1.2: bytes=56 Sequence=1 ttl=255 time=30 ms Reply from 10.0.1.2: bytes=56 Sequence=2 ttl=255 time=20 ms Reply from 10.0.1.2: bytes=56 Sequence=3 ttl=255 time=20 ms Reply from 10.0.1.2: bytes=56 Sequence=4 ttl=255 time=20 ms Reply from 10.0.1.2: bytes=56 Sequence=5 ttl=255 time=20 ms --- 10.0.1.2 ping statistics --- 5 packet(s) transmitted 5 packet(s) received 0.00% packet loss round-trip min/avg/max = 20/22/30 ms [R1]

R2

<R2>sys Enter system view, return user view with Ctrl+Z. [R2]ping -a 10.0.1.2 10.0.1.1 PING 10.0.1.1: 56 data bytes, press CTRL_C to break Reply from 10.0.1.1: bytes=56 Sequence=1 ttl=255 time=30 ms Reply from 10.0.1.1: bytes=56 Sequence=2 ttl=255 time=20 ms Reply from 10.0.1.1: bytes=56 Sequence=3 ttl=255 time=20 ms Reply from 10.0.1.1: bytes=56 Sequence=4 ttl=255 time=20 ms Reply from 10.0.1.1: bytes=56 Sequence=5 ttl=255 time=20 ms --- 10.0.1.1 ping statistics --- 5 packet(s) transmitted 5 packet(s) received 0.00% packet loss round-trip min/avg/max = 20/22/30 ms [R2]

LoopBack0 на R1 может взаимодействовать с LoopBack0 на R2.

Конфигурация на R1

[V200R003C00] # sysname R1 # snmp-agent local-engineid 800007DB03000000000000 snmp-agent # clock timezone China-Standard-Time minus 08:00:00 # portal local-server load portalpage.zip # drop illegal-mac alarm # set cpu-usage threshold 80 restore 75 # aaa authentication-scheme default authorization-scheme default accounting-scheme default domain default domain default_admin local-user admin password cipher %$%$K8m.Nt84DZ}e#<0`8bmE3Uw}%$%$ local-user admin service-type http # firewall zone Local priority 15 # interface GigabitEthernet0/0/0 ip address 10.0.12.1 255.255.255.0 # interface GigabitEthernet0/0/1 ip address 10.0.13.1 255.255.255.0 # interface GigabitEthernet0/0/2 # interface NULL0 # interface LoopBack0 ip address 10.0.1.1 255.255.255.255 # ip route-static 0.0.0.0 0.0.0.0 10.0.12.2 ip route-static 10.0.1.3 255.255.255.255 10.0.13.3 # user-interface con 0 authentication-mode password user-interface vty 0 4 user-interface vty 16 20 # wlan ac # return

Конфигурация на R2

[V200R003C00] # sysname R2 # snmp-agent local-engineid 800007DB03000000000000 snmp-agent # clock timezone China-Standard-Time minus 08:00:00 # portal local-server load portalpage.zip # drop illegal-mac alarm # set cpu-usage threshold 80 restore 75 # aaa authentication-scheme default authorization-scheme default accounting-scheme default domain default domain default_admin local-user admin password cipher %$%$K8m.Nt84DZ}e#<0`8bmE3Uw}%$%$ local-user admin service-type http # firewall zone Local priority 15 # interface GigabitEthernet0/0/0 ip address 10.0.12.2 255.255.255.0 # interface GigabitEthernet0/0/1 # interface GigabitEthernet0/0/2 ip address 10.0.23.2 255.255.255.0 # interface NULL0 # interface LoopBack0 ip address 10.0.1.2 255.255.255.255 # ip route-static 10.0.1.1 255.255.255.255 10.0.12.1 ip route-static 10.0.1.1 255.255.255.255 10.0.23.3 preference 100 ip route-static 10.0.1.3 255.255.255.255 10.0.23.3 # user-interface con 0 authentication-mode password user-interface vty 0 4 user-interface vty 16 20 # wlan ac # return

Конфигурация на R3

[V200R003C00] # sysname R3 # snmp-agent local-engineid 800007DB03000000000000 snmp-agent # clock timezone China-Standard-Time minus 08:00:00 # portal local-server load portalpage.zip # drop illegal-mac alarm # set cpu-usage threshold 80 restore 75 # aaa authentication-scheme default authorization-scheme default accounting-scheme default domain default domain default_admin local-user admin password cipher %$%$K8m.Nt84DZ}e#<0`8bmE3Uw}%$%$ local-user admin service-type http # firewall zone Local priority 15 # interface GigabitEthernet0/0/0 # interface GigabitEthernet0/0/1 ip address 10.0.13.3 255.255.255.0 # interface GigabitEthernet0/0/2 ip address 10.0.23.3 255.255.255.0 # interface NULL0 # interface LoopBack0 ip address 10.0.1.3 255.255.255.255 # ip route-static 10.0.1.1 255.255.255.255 10.0.13.1 ip route-static 10.0.1.2 255.255.255.255 10.0.23.2 # user-interface con 0 authentication-mode password user-interface vty 0 4 user-interface vty 16 20 # wlan ac # return

Вывод

В ходе лабораторной работы были изучены основные принципы IPv4-адресации и маршрутизации. Были настроены IPv4-адреса на интерфейсах маршрутизаторов и созданы loopback-интерфейсы, позволяющие использовать стабильные логические адреса. Также были рассмотрены принципы формирования прямых маршрутов и настройки статической маршрутизации. С помощью команды ping была проверена связность между узлами сети.